New Era Pump Systems Inc.

www.SyringePump.com

(631) 249-1392

PUMPTERM.EXE

NE-1000 Family of Programmable Syringe Pumps

RS-232 Network Terminal Emulator

This program is intended to be a basic demonstration of the RS-232 functionality

of the NE-1000 family of programmable pumps.  It allows communication in 'Basic' or 'Safe' mode.  In addition, command files, which may include pumping programs, can be downloaded to the pump network, allowing easy development and editing of pumping programs.

For best operation:  Copy all files to the directory:

C:\PUMPTERM

This program can be run from a DOS command line by entering the program name:

PUMPTERM

Also, a short-cut icon can be placed on the desktop to PUMPTERM.EXE

Startup parameters can be specified on the desktop icon by right-clicking the icon, selecting Properties, then the Program tab.  The program path is in field “Cmd Line:”.  At the end of the path name, enter any parameters

Example:  To communicate with Com port 3 and automatically load command file PPL.PPL:

Cmd Line:	C:\PUMPTERM\PUMPTERM.EXE /com=3 PPL.PPL

See DOS Command Line below for parameters that can be added to the short-cut icon, or entered on the DOS command line.

Different short-cut icons can be created for quick access to different Pumping Programs.

----------------------------------------------------------------------------

DOS Command Line:

PUMPTERM [ <param> ] [filename]

Filename				Command file

param =>

/ <option>			Option

<param>	[<param>]		Multiple parameters allowed

option => ADR	= {number}		Set Default Pump Network Address

COM	= {number}		Set RS-232 communications port

BAUD	= {number}		Set RS-232 baud rate

LOG	= {filename}		Specify log file name

Press enter after entering any data

Command Line		Send commands directly to pump network.

<F1>	Default Pump Network Address

<F2>	Safe Communications Mode

<F3>	Safe Communications Mode Timeout

<F4>	Auto-Update Mode

<F5>	Auto-Update Mode Timer

<F6>	Auto-Update Mode Command

<F7>	Upload command file to Pump Network

<F8>	Command file name

<F9> 	Log File status

<F10>	Log File Name

-----------------------------------------------------------------------

Command files (save file as text):

- Spaces in commands automatically parsed out

- Precede comments with a semi-colon ';'.

- Blank lines ignored

- Command file commands

{& name} in parameter is substituted for named phase

-----------------------------------------------------------------------

Command file commands:

(direct command)	Command sent directly to pump

SETUP [<param>]		Sets terminal program communication parameters

<param> =>

ADR = { ALL | n, [n] }

COM = { 1 | 2 | 3 | 4 }

BAUD = { n | AUTO }

PHASE [ name | n ]	Sets pump program's phase number to 'n'.

If number is not specified, the phase number is

automatically incremented.  'name' is assigned

to the current phase number, and can be referenced

in later commands as '&name'.

SAFE [nnn]		Set safe mode on or off (0, default), where 'nnn'

is the timeout

CHAIN	filename	Continues execution with “filename” Pumping Program

CALL	filename	Calls sub-file “filename” Pumping Program

After execution of “filename” the next command after the CALL command will be executed

LOG { ON | OFF | NEW }	Controls log file

TIMESTAMP		Writes the current time to the open log file.

EXIT			Exits terminal program